# Module 1: Explaining Network Topologies — Guided Notes

## Module Overview

### Module Objectives

By the end of this module, you should be able to:

- Explain network types and characteristics.

Describe common network types, including LANs and WANs, and explain how networks differ in size, purpose, structure, and connectivity.

- Compare and contrast OSI model layers.

Identify the seven layers of the Open Systems Interconnection model and explain the protocols, devices, addressing, and data units associated with each layer.

- Configure SOHO networks.

Explain how small office/home office routers combine networking functions, including switching, wireless access, routing, DHCP, modem connectivity, and firewall security.

- Explain CompTIA’s troubleshooting methodology.

Apply a structured, seven-step process to identify, diagnose, resolve, verify, and document network problems.

# Lesson 1.1: Networking Overview

## Certification Exam Objectives Covered

- 1.2: Compare and contrast networking appliances, applications, and functions.

- 1.6: Compare and contrast network topologies, architectures, and types.

## Networking Concepts

### Client-Server Network

Definition:

A client-server network is a network in which some nodes act as clients. Clients request services or resources from other computers called servers.

Characteristics:

- A client is a computer or device that requests a service.

- A server is a computer or device that provides a service or resource.

- Servers may provide:

  - File storage

  - Printing

  - Email

  - Web pages

  - Authentication

  - Databases

  - Applications

- A server usually has more processing power, storage, availability, or security controls than a typical client.

- One server can support many clients.

- Network administrators can manage users, permissions, backups, and security from centralized systems.

Examples of use:

- A school file server providing shared folders to teachers and students

- A web server delivering web pages to browsers

- An email server handling messages for an organization

- A database server supporting a student-information system

- An authentication server verifying user credentials

### Peer-to-Peer Network

Definition:

A peer-to-peer network is a network in which each host acts as both a client and a server.

Characteristics:

- Every connected computer can request resources.

- Every connected computer can provide resources.

- There is no requirement for a dedicated central server.

- Devices may share files, folders, printers, or other services directly with one another.

- Each user or device may be responsible for managing its own security and shared resources.

- Peer-to-peer networks are generally better suited to small networks with limited management requirements.

Advantages:

- Simple to set up

- Inexpensive because a dedicated server may not be needed

- Easy to share resources directly between devices

- No single server is required for basic operation

Disadvantages:

- Difficult to manage as the network grows

- Security settings may be inconsistent between devices

- Backups may be decentralized

- Performance depends on the individual peer devices

- If a device is turned off, its shared resources may be unavailable

- There is no centralized authentication or resource management

## Network Types

### Local Area Network (LAN)

Definition:

A local area network is a group of devices connected in a single location.

Characteristics:

- Geographic scope:

  - Usually a home, classroom, office, school building, or campus

  - Covers a relatively limited geographic area

- Typical devices included:

  - Computers

  - Servers

  - Printers

  - Network switches

  - Wireless access points

  - Routers

  - Network-attached storage devices

  - Internet of Things devices

- Common uses:

  - Sharing files and printers

  - Accessing local applications

  - Connecting users to school or business resources

  - Providing Internet access

  - Supporting voice, video, and data communication

A LAN may use wired Ethernet, wireless networking, or both.

### Wide Area Network (WAN)

Definition:

A wide area network is a network of networks connected by long-distance links.

Characteristics:

- Geographic scope:

  - May connect buildings, cities, states, countries, or continents

  - Usually covers a larger area than a LAN

- Connection methods:

  - Fiber-optic links

  - Leased lines

  - Broadband connections

  - Cellular networks

  - Satellite links

  - Virtual private networks

  - Internet connections

- Common examples:

  - A school district connecting multiple campuses

  - A company connecting branch offices

  - The Internet

  - A regional healthcare network connecting hospitals and clinics

  - A cloud-connected business network

A WAN commonly connects multiple LANs.

## Network Topologies

### What Is Network Topology?

Network topology is the physical or logical arrangement of devices, links, and connections in a network.

- Physical topology describes how devices and cables are physically arranged.

- Logical topology describes how data moves through the network, regardless of the physical arrangement.

Topology affects:

- Network performance

- Cost

- Reliability

- Scalability

- Troubleshooting

- How a failure affects other devices

### Star Topology

Description:

In a star topology, each device connects to a central networking device. The end devices do not normally connect directly to one another.

Central device:

- Usually an Ethernet switch

- May also be a hub or wireless access point, depending on the network

Advantages:

- Easy to install and manage

- A failure in one cable usually affects only one device

- Easy to add or remove devices

- Centralized monitoring and troubleshooting

- Good performance when a switch is used

- A failed endpoint does not normally bring down the entire network

Disadvantages:

- Failure of the central device can affect the entire network

- Requires more cabling than some other topologies

- The central device may become a performance bottleneck

- Installation costs may be higher because every device needs a connection to the center

Common applications:

- Modern Ethernet LANs

- Classroom networks

- Office networks

- Home networks

- Wireless networks using a central access point

### Mesh Topology

Description:

In a mesh topology, devices interconnect with multiple links. A mesh provides multiple possible paths between network devices.

#### Types

Full mesh:

- Every device has a direct connection to every other device.

- Provides maximum redundancy.

- Requires a large number of links as the network grows.

Partial mesh:

- Only some devices have multiple direct connections.

- Critical devices may have redundant links, while less important devices have fewer connections.

- Provides a balance between reliability and cost.

Advantages:

- High availability

- Multiple paths for data

- Strong redundancy

- Network communication may continue if one link fails

- Useful for critical systems and backbone connections

Disadvantages:

- Expensive to install

- Requires more cabling, ports, and hardware

- More difficult to configure and manage

- Troubleshooting can be complex

- Full mesh becomes increasingly impractical as the number of devices increases

Common applications:

- Internet and service-provider backbones

- Wireless mesh networks

- Data-center networks

- Wide area networks

- Military, emergency, and mission-critical communication systems

# Lesson 1.2: OSI Model Concepts

## Certification Exam Objective Covered

- 1.1: Explain concepts related to the Open Systems Interconnection (OSI) reference model.

## Open Systems Interconnection (OSI) Model

### Purpose of the OSI Model

The Open Systems Interconnection model, commonly called the OSI model, is a conceptual framework that divides network communication into seven functional layers.

The OSI model exists to:

- Provide a common language for discussing networking

- Explain how data moves from one device to another

- Separate networking functions into manageable sections

- Help different manufacturers create interoperable products

- Simplify network design and troubleshooting

- Identify where a failure or protocol operates

- Show how data is encapsulated and transmitted

The OSI model is a reference model. Actual network protocols may combine functions from multiple OSI layers.

### The Seven Layers

Listed from top to bottom:

| Layer | Name | Primary Purpose |
| --- | --- | --- |
| 7 | Application | Provides network services directly to user applications |
| 6 | Presentation | Formats, translates, encrypts, and compresses data |
| 5 | Session | Establishes, manages, and terminates communication sessions |
| 4 | Transport | Provides end-to-end delivery, reliability, flow control, and port addressing |
| 3 | Network | Provides logical addressing and routing between networks |
| 2 | Data Link | Transfers frames between devices on the same logical segment |
| 1 | Physical | Transmits signals representing bits across a physical medium |

A helpful way to remember the layers from top to bottom is:

All People Seem To Need Data Processing

- A — Application

- P — Presentation

- S — Session

- T — Transport

- N — Network

- D — Data Link

- P — Physical

## Data Encapsulation and Decapsulation

### Network Protocol

Definition:

A network protocol is a set of rules for exchanging data.

Protocols define such things as:

- How devices identify one another

- How data is formatted

- How data is addressed

- How errors are handled

- How devices establish communication

- How data is transmitted and received

Examples include Ethernet, Internet Protocol, Transmission Control Protocol, User Datagram Protocol, Hypertext Transfer Protocol, and Domain Name System.

### Addressing

Definition:

Addressing describes where data messages should go.

Purpose:

- Identifies the source of data

- Identifies the destination of data

- Helps devices determine how to forward information

- Allows communication between the correct applications, hosts, and networks

Different layers use different types of addressing:

- Layer 2 commonly uses MAC addresses.

- Layer 3 commonly uses IP addresses.

- Layer 4 uses port numbers to identify applications or services.

### Encapsulation

Definition:

Encapsulation describes how data messages should be packaged.

Process:

- An application creates data.

- As the data moves down the OSI layers, each layer adds its own control information.

- This information is usually placed in a header; some layers may also add a trailer.

- The Layer 4 data becomes a segment or datagram.

- The Layer 3 data becomes a packet.

- The Layer 2 data becomes a frame.

- The Layer 1 device transmits the frame as signals representing bits.

Encapsulation is similar to placing a message inside progressively larger envelopes, with each envelope containing information needed by a different layer.

### Decapsulation

Process:

- The receiving device detects signals at Layer 1.

- The signals are converted into bits and passed to Layer 2.

- Layer 2 examines the frame and removes its header and trailer.

- Layer 3 examines the packet and removes its network-layer information.

- Layer 4 processes the segment or datagram and delivers the data to the correct port.

- The upper layers interpret, decrypt, decompress, or manage the data.

- The application receives the original information.

Decapsulation is the reverse of encapsulation.

## Layer 1 — Physical Layer

### Function

The Physical layer transmits and receives signals representing bits of data.

It handles the physical transmission of information through media such as:

- Copper cable

- Fiber-optic cable

- Radio waves

- Other physical communication media

### What It Specifies

- Physical topology:

The physical arrangement of devices, cables, connectors, and other components.

- Physical interface:

The mechanical and electrical or optical characteristics of a connection, such as connector type, pin arrangement, voltage, or light transmission.

- Signaling:

How bits are represented as electrical, optical, or radio signals.

### Devices Operating at Layer 1

- Transceiver:

A device that can transmit and receive signals. The term combines “transmitter” and “receiver.”

- Repeater:

Receives a signal, regenerates or strengthens it, and retransmits it to extend the distance of a network connection.

- Hub:

A basic Layer 1 device that repeats incoming signals out to multiple ports. It does not intelligently forward traffic to a specific destination.

- Media converter:

Converts signals from one transmission medium to another, such as copper Ethernet to fiber-optic Ethernet.

### Data Unit

The Physical layer transmits bits.

## Layer 2 — Data Link Layer

### Function

The Data Link layer transfers data between nodes on the same logical segment.

It also:

- Performs an encapsulation function

- Organizes bits from the Physical layer into frames

- Adds control information in header fields

- Uses hardware addresses to identify devices on a local segment

### Key Responsibilities

- Encapsulation function:

The Data Link layer organizes bits received from Layer 1 into a structured unit called a frame.

- Frame structure:

A frame generally contains:

  - A header

  - Source and destination MAC addresses

  - Control information

  - A payload containing the Layer 3 packet

  - A trailer that may contain error-detection information

The frame header provides information needed for local delivery. The payload contains the data being carried for higher layers.

### Devices Operating at Layer 2

- Network Interface Card (NIC):

Provides a device with a network connection and a hardware MAC address.

- Bridge:

Connects network segments and forwards or filters frames based on MAC addresses.

- Switch:

Connects devices on a LAN and forwards frames to the appropriate port using a MAC address table.

- Wireless access point (AP):

Provides wireless connectivity and connects wireless stations to a wired or wireless network.

### Data Unit

Frame

### Important Addressing

MAC address:

A Media Access Control address is a hardware address associated with a network interface. It is used for local network communication.

## Layer 3 — Network Layer

### Function

The Network layer moves data around an internetwork using logical network and host IDs.

### Key Responsibilities

- Logical addressing:

Uses logical addresses, such as IP addresses, to identify networks and hosts.

- Routing:

Moves packets between different networks.

- Path determination:

Determines the best available path for forwarding data toward its destination.

### Appliances and Devices Operating at Layer 3

#### Router

Function:

A router connects different networks and makes forwarding decisions based on logical addresses and routing information.

A router may connect:

- A private LAN to the public Internet

- One office network to another

- Multiple subnets within an organization

- A local network to a WAN

#### Basic Firewall

Function:

A basic firewall operates by enforcing an access control list, or ACL.

An ACL contains rules that determine whether particular traffic should be:

- Allowed

- Denied

- Logged

- Restricted based on address, protocol, port, or direction

### Data Unit

Packet

### Important Addressing

IP address

An IP address identifies a network interface logically and helps routers deliver packets between networks.

## Layer 4 — Transport Layer

### Function

The Transport layer is the end-to-end or host-to-host layer.

It supports communication between applications running on source and destination hosts.

### Key Responsibilities

- Port number assignment:

Port numbers identify the type of network application or service receiving data. For example, a web server and an email server can run on the same host while using different port numbers.

- Connection management:

Some transport protocols establish, maintain, and terminate a connection before transferring data.

- Flow control:

Regulates the rate of transmission so a sender does not overwhelm a receiver.

- Reliability:

Some protocols provide acknowledgments, sequencing, retransmission, and error recovery.

- Segmentation and reassembly:

Breaks large amounts of data into smaller units and reassembles them at the destination.

### Devices Operating at Layer 4

- Multilayer switches:

Switches that can make forwarding decisions using information from multiple OSI layers, including transport-layer ports.

- Security appliances:

Devices that inspect and control traffic based on ports, connections, applications, and security rules.

### Data Unit

- Segment: Used with TCP

- Datagram: Used with UDP

## Upper Layers: Layers 5–7

### Layer 5 — Session Layer

Function:

The Session layer establishes, manages, and terminates communication sessions between applications.

Responsibilities:

- Opens communication sessions

- Maintains sessions

- Coordinates dialog between systems

- Controls which device transmits at a given time

- Synchronizes communication

- Terminates sessions when communication is complete

### Layer 6 — Presentation Layer

Function:

The Presentation layer ensures that data is represented in a format the receiving system can understand.

Responsibilities:

- Data translation

- Character-code conversion

- Data formatting

- Encryption and decryption

- Compression and decompression

### Layer 7 — Application Layer

Function:

The Application layer provides network services directly to applications used by people or other software.

Responsibilities:

- Provides access to network resources

- Supports file transfers

- Supports web browsing

- Supports email communication

- Supports name resolution

- Provides network service requests and responses

Examples of protocols:

- HTTP and HTTPS for web communication

- DNS for name resolution

- SMTP for sending email

- IMAP and POP3 for retrieving email

- FTP and SFTP for file transfer

- SSH for secure remote administration

- DHCP for automatic IP configuration

## OSI Model Summary

| Layer | Name | Primary Function | Data Unit | Key Devices or Technologies |
| --- | --- | --- | --- | --- |
| 7 | Application | Provides network services to applications | Data | Web, email, DNS, file-transfer services |
| 6 | Presentation | Translates, encrypts, decrypts, compresses, and decompresses data | Data | Encryption and data-format systems |
| 5 | Session | Establishes, manages, and terminates sessions | Data | Session-management functions |
| 4 | Transport | Provides end-to-end delivery, port addressing, flow control, and reliability | Segment or datagram | Multilayer switches, security appliances |
| 3 | Network | Provides logical addressing and routing between networks | Packet | Routers and basic firewalls |
| 2 | Data Link | Transfers frames across the same logical segment | Frame | NICs, bridges, switches, wireless APs |
| 1 | Physical | Transmits signals representing bits | Bits | Transceivers, repeaters, hubs, media converters |

### Layer Addressing Review

- Layer 2: MAC address

- Layer 3: IP address

- Layer 4: Port number

### Encapsulation Review

Application data becomes a:

- Segment or datagram at Layer 4

- Packet at Layer 3

- Frame at Layer 2

- Bit stream at Layer 1

# Lesson 1.3: SOHO Networks

## Certification Exam Objective Covered

- 1.1: Explain concepts related to the Open Systems Interconnection (OSI) reference model.

## SOHO Routers

### Definition

SOHO stands for Small Office/Home Office.

A SOHO router is an intermediate system powering SOHO networks.

### Function

A SOHO router combines several networking functions into one device, including:

- Ethernet switching

- Wireless access

- Routing

- DHCP services

- Firewall security

- WAN connectivity

- Sometimes modem functionality

### Primary Role

The primary role of a SOHO router is to forward traffic between the LAN and WAN.

- The LAN side connects local devices.

- The WAN side connects to an ISP or another external network.

- The router examines traffic and determines whether it should remain on the local network or be forwarded externally.

## Physical Layer Functions: Layer 1

### RJ45 Ports: LAN Ports

Purpose:

RJ45 ports implement a local cabled network.

What they connect:

- Desktop computers

- Laptops with wired adapters

- Printers

- Network switches

- Servers

- Other Ethernet-enabled devices

RJ45 ports use Ethernet cabling, commonly twisted-pair copper cable.

### Radio Antennae

Purpose:

Radio antennae transmit and receive wireless signals.

What they enable:

- Wireless devices to connect to the LAN

- Wi-Fi communication

- Wireless access without physical Ethernet cables

- Communication between wireless clients and the SOHO router

### Modem

Purpose:

A modem provides the connection between the SOHO network and the ISP’s network.

What it connects to:

- The WAN service provided by an Internet Service Provider

- A broadband, telephone, cable, fiber, cellular, or other provider network

Some SOHO routers contain an integrated modem. In other setups, the modem is a separate device connected to the router’s WAN port.

## Data Link Layer Functions: Layer 2

### Ethernet Switch

Purpose:

The Ethernet switch connects RJ45 jacks internally.

What it connects:

- Wired LAN devices to one another

- Wired devices to the routing function inside the SOHO device

- Multiple Ethernet ports into one local network

The switch forwards frames using MAC addresses.

### Wireless Access Point

Purpose:

A wireless access point acts as a wireless hub.

What it enables:

- Wireless stations to form a wireless network

- Wireless devices to communicate with one another

- Wireless devices to communicate with wired LAN devices

- Wireless clients to access services through the router

## Network Layer Functions: Layer 3

### Routing Function

Purpose:

The routing part of a SOHO router makes forwarding decisions.

What it routes between:

- The local private network

- The public Internet

- The LAN and WAN interfaces

- Separate local networks or subnets, when configured

The router examines logical addressing information and determines where packets should be sent.

### DHCP Server

DHCP stands for:

Dynamic Host Configuration Protocol

Purpose:

A SOHO router commonly runs a DHCP server to automatically configure hosts that connect to the network.

What it allocates:

- A unique IP address

- A subnet mask

- A default gateway

- DNS server information

- Other network configuration information

DHCP reduces the need to configure each device manually.

## Transport and Application Layer Functions and Security

### Network Security

Definition:

Network security is the process of allowing or preventing devices, users, and services from using the network.

Security controls may consider:

- Source and destination addresses

- Users and devices

- Applications

- Protocols

- Port numbers

- Connection direction

- Time or location

### SOHO Router Firewall

What it implements:

- A firewall that filters network traffic

- Access-control rules

- Security policies for LAN and WAN traffic

How it is configured:

A firewall can be configured with rules specifying behavior for each port.

Rules may:

- Allow traffic

- Block traffic

- Allow only certain sources or destinations

- Restrict inbound connections

- Restrict outbound connections

- Log connection attempts

What it controls:

- Which devices may access the network

- Which services may be used

- Which ports may receive traffic

- Whether traffic can enter from or leave toward the Internet

## The Internet

### How SOHO Networks Connect

- Connection point: The WAN interface

- Connection method: The SOHO network connects to an ISP through a WAN service. The slide identifies the Public Switched Telephone Network, or PSTN, as one method used to facilitate Internet access.

### PSTN

Stands for:

Public Switched Telephone Network

Purpose:

The PSTN is the traditional worldwide telephone network. It can provide a communications path used by some Internet-access technologies, particularly telephone-based or digital subscriber services.

### Internet Service Providers

ISPs provide connectivity between a customer’s network and the Internet.

An ISP may provide:

- Internet access

- WAN links

- IP address assignment

- DNS services

- Modem or optical-network equipment

- Technical support

- Additional services such as email, hosting, or security

### Internet Standards Organizations

#### IANA

Stands for:

Internet Assigned Numbers Authority

Responsibilities:

- Coordinates global Internet number resources

- Helps manage IP address allocations

- Coordinates protocol parameters

- Maintains registries for protocol-related values

- Coordinates the DNS root zone in cooperation with other organizations

#### IETF

Stands for:

Internet Engineering Task Force

Responsibilities:

- Develops and maintains Internet standards

- Publishes technical specifications and Request for Comments documents

- Works on protocols and technologies that support Internet interoperability

- Uses working groups and open technical collaboration

## Binary and Hexadecimal Number Systems

### Binary: Base 2

Definition:

Binary is a base-2 number system.

Possible digit values:

The only possible digits are 0 and 1.

Place positions:

Each position represents the next power of 2. In an 8-bit value, the place values from left to right are:

128, 64, 32, 16, 8, 4, 2, 1

Byte/Octet:

A byte, also called an octet in networking, is an 8-bit value.

Binary is important in networking because computers and network devices represent data using bits.

### Hexadecimal: Base 16

Definition:

Hexadecimal, or hex, is a base-16 number system.

Possible digit values:

Hexadecimal uses:

- Numerals from 0 through 9

- Letters A through F

The letters represent values:

- A = 10

- B = 11

- C = 12

- D = 13

- E = 14

- F = 15

Purpose in networking:

- Provides a shorter, more readable representation of binary values

- Commonly represents MAC addresses

- Used in IPv6 addresses

- Used in protocol fields, memory addresses, and diagnostic output

- Each hexadecimal digit represents four binary bits

## Number System Conversion Table

| Decimal | Hexadecimal | Binary |
| --- | --- | --- |
| 0 | 0 | 0000 |
| 1 | 1 | 0001 |
| 2 | 2 | 0010 |
| 3 | 3 | 0011 |
| 4 | 4 | 0100 |
| 5 | 5 | 0101 |
| 6 | 6 | 0110 |
| 7 | 7 | 0111 |
| 8 | 8 | 1000 |
| 9 | 9 | 1001 |
| 10 | A | 1010 |
| 11 | B | 1011 |
| 12 | C | 1100 |
| 13 | D | 1101 |
| 14 | E | 1110 |
| 15 | F | 1111 |

### Conversion Practice

Complete the following:

- Decimal 7 = Hexadecimal _____ = Binary _____

- Decimal 10 = Hexadecimal _____ = Binary _____

- Decimal 15 = Hexadecimal _____ = Binary _____

- Hexadecimal C = Decimal _____ = Binary _____

- Binary 0110 = Decimal _____ = Hexadecimal _____

- Binary 1111 = Decimal _____ = Hexadecimal _____

# Lesson 1.4: Troubleshooting Methodology

## Certification Exam Objective Covered

- 5.1: Explain the troubleshooting methodology.

## CompTIA Network+ Troubleshooting Methodology

The seven steps are:

- Identify the problem

- Establish a theory of probable cause

- Test the theory to determine the cause

- Establish a plan of action to resolve the problem and identify potential effects

- Implement the solution or escalate as necessary

- Verify full system functionality and implement preventive measures if applicable

- Document findings, actions, outcomes, and lessons learned throughout the process

A structured methodology prevents technicians from making random changes and helps preserve evidence, reduce downtime, and avoid creating additional problems.

## Step 1: Identify the Problem

### Gather Information

Begin by collecting facts before changing the system.

- Identify the location and scope of the problem.

- Check system documentation.

- Check recent job logs or consult other technicians.

- Use vendor support sites and forums.

- Determine which users, devices, services, or network segments are affected.

- Record the time the problem began and any error messages.

### Question Users

Users may provide important information about symptoms, timing, and recent changes.

#### Open Questions

Definition:

Open questions invite the user to provide an explanation or detailed response.

Purpose:

- Gather complete information

- Encourage the user to describe what happened

- Discover symptoms the technician may not have observed

- Learn about recent changes or patterns

Examples:

- “What happened when the problem started?”

- “Can you describe what you were trying to do?”

- “What have you noticed about the network connection?”

- “When does the problem occur?”

- “What changed before the issue appeared?”

#### Closed Questions

Definition:

Closed questions require a yes/no answer or a response from a fixed set of choices.

Purpose:

- Confirm specific facts quickly

- Narrow the range of possible causes

- Verify whether a particular symptom exists

- Save time after open questions have established the general situation

Examples:

- “Can you access the Internet?”

- “Is the network cable connected?”

- “Did the problem begin today?”

- “Are other users affected?”

- “Does the link light appear on the switch port?”

## Identify Problem Symptoms

### Identify Symptoms

- Make a physical inspection.

- Check system logs or diagnostic software.

- Duplicate the problem on the user’s system or a test system.

- Record error messages and warning indicators.

- Check link lights, cable connections, power status, and device displays.

- Determine whether the issue affects connectivity, performance, authentication, or a specific application.

### Determine if Anything Has Changed

Ask:

- “Did it ever work?”

- “What has changed since it was last working?”

Possible changes include:

- New hardware

- Software updates

- Configuration changes

- New cables

- Account or password changes

- Firewall rules

- Network maintenance

- Environmental changes

- Power outages

- New users or devices

### Approach Multiple Problems Individually

Why:

Symptoms might be caused by more than one problem.

Method:

Treat each issue as a separate case. Document and troubleshoot each symptom independently before assuming that all symptoms have one cause.

For example, a user might have both:

- A disconnected network cable

- An unrelated DNS configuration problem

Solving one issue may not solve the other.

## Step 2: Establish a Theory of Probable Cause

### Top-to-Bottom/Bottom-to-Top OSI Model Approach

Definition:

This is a methodical validation of network components by testing at each layer of the OSI model in sequence.

Method:

- Start at Layer 1 and work upward, or start at Layer 7 and work downward.

- Test each layer before moving to the next.

- Use evidence to determine whether each layer is functioning correctly.

- Stop when the likely source of the problem is identified.

When to use:

- When the problem is broad or unclear

- When several network layers may be involved

- When a systematic, complete diagnosis is needed

- When you want to avoid overlooking a lower- or higher-layer problem

### Divide and Conquer Approach

Definition:

The divide and conquer approach starts with the layer most likely causing the problem and then works down or up depending on what tests reveal.

Method:

- Select a likely point in the OSI model.

- Test that layer or a nearby layer.

- If the test passes, move toward another layer.

- If the test fails, investigate that layer and the components associated with it.

When to use:

- When symptoms strongly suggest a particular layer

- When time is limited

- When the problem can quickly be narrowed to a specific area

- When testing a known boundary, such as the connection between a LAN and WAN

## Step 3: Test the Theory to Determine the Cause

### If the Theory Is Confirmed

- Proceed to the planning step.

- Identify the specific cause.

- Record the evidence supporting the theory.

- Consider whether additional problems may remain.

### If the Theory Is Not Confirmed

- Develop and test a new theory.

- Return to the available evidence.

- Avoid forcing the symptoms to fit the original theory.

- Test one likely cause at a time when possible.

### When to Escalate

Escalate the problem when:

- It is beyond your knowledge or ability to troubleshoot.

- It falls under a system warranty.

- The scope is large and/or the solution requires major network reconfiguration.

- A customer becomes difficult or abusive.

### Escalation Alternatives

- Internal senior staff or knowledge experts

- Suppliers and manufacturers

- Support contractors or consultants

- Other authorized technical support resources

## Step 4: Establish a Plan of Action

### Three Solutions to Any Problem

- Repair

Fix or restore the existing component, configuration, or service.

- Replace

Substitute a faulty or unsuitable component with a working one.

- Accept

Decide that the issue will not be corrected because the cost, time, risk, or impact of correction is not justified.

### Considerations

- Assess the cost and time required.

- Consider potential effects on the rest of the system.

- Determine whether the solution may cause downtime.

- Identify security or performance consequences.

- Confirm authorization before making changes.

- Plan a rollback in case the change does not work.

- Consider whether users or other services will be affected.

## Step 5: Implement the Solution

### Implement Directly When

Implement the solution directly when the change is within your authority, knowledge, and technical ability and does not require a formal change process.

Examples may include:

- Reconnecting a loose cable

- Replacing a damaged patch cable

- Correcting a simple documented configuration

- Restarting an approved service

### Follow a Change Management Plan When

Follow a change management plan when the solution requires a change to the system or network environment.

A change management plan may include:

- Approval

- Scheduling

- Communication

- Risk assessment

- Backup or rollback procedures

- Testing

- Documentation

### Escalate to Senior Personnel When

Escalate to senior personnel when you do not have authorization to implement a solution.

Also escalate when:

- The change is high risk.

- The issue affects many users.

- The solution requires specialized expertise.

- The system is under warranty or support agreement.

- The change could cause significant downtime.

## Step 6: Verify the Solution

### Verification Steps

- Identify the results and effects of the solution.

- Validate that the solution fixes the reported problem.

- Validate that the system continues to function normally.

- Obtain the customer’s acceptance.

- Eliminate factors that may cause the problem to recur.

- Confirm that related services and devices still work.

- Monitor the system when appropriate.

- Implement preventive measures if applicable.

### Key Validations

Ask:

- Is the original symptom gone?

- Can the user complete the original task?

- Can other users still access network resources?

- Are performance and security acceptable?

- Did the change create any new errors?

- Does the configuration survive a restart?

- Has the customer or user confirmed that the issue is resolved?

Verification is not complete merely because the original error disappears. Full system functionality must be checked.

## Step 7: Document Findings, Actions, and Outcomes

### Where to Document

Document troubleshooting within a ticket system.

### What to Include

Add a complete description of:

- The reported problem

- Affected users, devices, and locations

- Symptoms and error messages

- Diagnostic tests performed

- The probable cause

- Findings

- Actions taken

- Changes made

- Results

- Final outcome

- Preventive measures

- Lessons learned

- Any escalation or vendor contact

### Documentation Best Practices

- Write clearly and concisely.

- Use objective, factual language.

- Record events in the order they occurred.

- Include relevant times, settings, and test results.

- Avoid unexplained abbreviations.

- Do not assign blame.

- Record what worked and what did not work.

Why:

Troubleshooting logs may be presented to customers and others. Clear documentation supports future technicians, audits, training, accountability, and recurring-problem analysis.

# Vocabulary and Acronym List

## Vocabulary

Access point — A device that provides wireless network connectivity and bridges wireless clients to a wired or wireless network.

ACL — A rule set that allows, denies, logs, or restricts traffic based on addresses, protocols, ports, direction, or other criteria.

Binary — A base-2 number system that uses only 0 and 1.

Bridge — A Layer 2 device that connects network segments and forwards or filters frames using MAC addresses.

Byte — A group of 8 bits.

Client — A computer or device that requests services or resources from a server.

Client-server network — A network in which clients request services from dedicated servers.

Datagram — A Layer 4 data unit commonly associated with UDP and generally sent without a connection-oriented delivery guarantee.

Decapsulation — The process of removing headers and trailers as received data moves up the protocol stack.

Encapsulation — The process of adding control information as data moves down the protocol stack.

Firewall — A security system that filters network traffic according to defined rules.

Frame — The Layer 2 data unit that carries a packet across a local logical segment.

Hexadecimal — A base-16 number system using 0–9 and A–F.

Hub — A Layer 1 device that repeats incoming signals to multiple ports without intelligently selecting a destination.

IP address — A logical address that identifies a network interface and supports delivery between networks.

LAN — A network connecting devices within a limited geographic area, such as a home, office, or campus.

Logical topology — The arrangement that describes how data moves through a network.

MAC address — A hardware address associated with a network interface and used for local network communication.

Mesh topology — A topology in which devices have multiple interconnections and alternate paths.

Network topology — The physical or logical arrangement of devices, links, and connections.

NIC — A hardware interface that connects a device to a network and provides a MAC address.

Octet — An 8-bit value; in networking, the term commonly refers to one byte of an IP address.

OSI model — A seven-layer reference framework for describing network communication functions.

Packet — The Layer 3 data unit that contains logical addressing information for routing.

Peer-to-peer network — A network in which each host can act as both a client and a server.

Port number — A Layer 4 identifier used to direct traffic to a specific application or service.

Protocol — A defined set of rules for formatting, addressing, transmitting, receiving, and processing data.

Router — A Layer 3 device that connects networks and forwards packets using logical addresses and routing information.

Segment — A Layer 4 data unit commonly associated with TCP.

Server — A computer or device that provides services or resources to clients.

Star topology — A topology in which each endpoint connects to a central device, usually a switch or access point.

Switch — A Layer 2 device that forwards frames to the appropriate port using a MAC address table.

WAN — A network that connects separate networks over large geographic areas using long-distance links.

## Acronyms

ACL = Access Control List

AP = Access Point

DHCP = Dynamic Host Configuration Protocol

DNS = Domain Name System

FTP = File Transfer Protocol

HTTP = Hypertext Transfer Protocol

HTTPS = Hypertext Transfer Protocol Secure

IANA = Internet Assigned Numbers Authority

IETF = Internet Engineering Task Force

IMAP = Internet Message Access Protocol

IP = Internet Protocol

LAN = Local Area Network

MAC = Media Access Control

NIC = Network Interface Card

OSI = Open Systems Interconnection

POP3 = Post Office Protocol version 3

PSTN = Public Switched Telephone Network

RJ45 = Registered Jack 45

SFTP = SSH File Transfer Protocol

SMTP = Simple Mail Transfer Protocol

SOHO = Small Office/Home Office

SSH = Secure Shell

TCP = Transmission Control Protocol

UDP = User Datagram Protocol

WAN = Wide Area Network